## Requirements for the modeling the preliminary list of interventions

| ID | Intervention | Short Description/Info | A - Location and shape Information required | B – Shape + Hydrological Parameters required |
| --- | --- | --- | --- | --- |
| M: Mountains |  |  |  |  |
| M1 | Check Dam Delay Action Dams | Small barriers built across valleys to slow runoff, trap sediment, and enhance groundwater recharge during high-flow events. | - Weir / wall represented as vector polyline - Crest level of the Weir above m.s.l (in meters) - An outline/polygon of expected ponded surface will be useful for validating mesh modification - Note: Any associated canal abstraction / diversions should be mentioned but may not be modeled due to scale | - CN values and Manning’s n if changes are expected. - Spatial extent (vector polygon) if the location of change is different from location polygon in A. - Weir / wall represented as vector polyline - Crest level of the Weir above m.s.l (in meters) - Note: Any associated canal abstraction / diversions should be mentioned but may not be modeled due to scale |
| M2 | Infiltration Galleries (CCTs) | Shallow contour trenches constructed along slopes to capture rainfall, reduce erosion, and increase soil moisture. | - Vector Polygon marking region where infiltration galleries are to be implemented (can be delineated as one contiguous polygon for each mountain slope) - Total effective surface storage / pondage resulting from this - Note: cannot be modeled as a single gallery / trench or represented using a vector polyline | - CN values and Manning’s n if changes are expected. - Total effective surface storage / pondage resulting from this - Note: cannot be modeled as a single gallery / trench or represented using a vector polyline |
| M3 | Small Ponds | Small storage ponds in upper catchments used to collect rainwater for livestock, supplemental irrigation, and recharge. | - Weir / wall represented as vector polyline - Crest level of the Weir above m.s.l (in meters) - An outline/polygon of expected ponded surface will be useful for validating mesh modification | - CN values and Manning’s n if changes are expected. - Crest level of the Weir above m.s.l (in meters) - An outline/polygon of expected ponded surface will be useful for validating mesh modification |
| M4 | Afforestation | Planting trees on degraded slopes to stabilize soil, reduce surface runoff, and enhance watershed resilience. | - Vector Polygon marking region where intervention is to be implemented | - CN values and Manning’s n if changes are expected. - Note: Provide vector Polygon marking region where intervention is to be implemented |
| M5 | Slope Stabilizers / Cascading | Structural or vegetative measures installed along vulnerable slopes to reduce landslides and dissipate runoff energy. | - Vector Polygon marking region where intervention is to be implemented | - CN values and Manning’s n if changes are expected. - Note: Provide vector Polygon marking region where intervention is to be implemented |
| M6 | Community-Based Ecosystem Adaptation | Locally driven watershed interventions such as soil conservation, vegetation planting, and water harvesting designed and maintained by communities. | - Vector Polygon marking region where intervention is to be implemented | - CN values and Manning’s n if changes are expected. - Note: Provide vector Polygon marking region where intervention is to be implemented |
| M7 | Flood Diversion Bunds | Earthen bunds constructed to redirect peak flows away from settlements and agricultural lands. | - vector polyline + proposed name of the embankment | - Elevation of the embankement - Vector polyline + proposed name of the embankment |
| M8 | Sustainable Spate Irrigation System | Traditional flood-based irrigation using controlled diversion of hill-torrents to irrigate command areas sustainably | - Not modeled | - Default: No hydrological implication |
| M9 | Bank Vegetation & Erosion Control | Use of grasses, shrubs, or bio-engineering to stabilize banks, reduce erosion, and improve channel health. | - Vector Polygon marking region where intervention is to be implemented - In case of existing embankment, vector polyline + name of the existing embankment | - CN values and Manning’s n if changes are expected. - Vector Polygon marking region where intervention is to be implemented - In case of existing embankment, vector polyline + name of the existing embankment |
| M10 | Gabion Structures | Rock-filled wire mesh structures used to protect banks, prevent scouring, and strengthen vulnerable channel sections. | - Weir / wall represented as vector polyline - Crest level of the Weir above m.s.l (in meters) Note: Sediment transport and channel training cannot be modeled | - Default: No hydrological implication - Optional: CN values and Manning’s n (only if changes are expected). - Weir / wall represented as vector polyline - Crest level of the Weir above m.s.l (in meters) - Note: Sediment transport and channel training cannot be modeled |
| M11 | Gabar Dams (traditional rainwater harvesting) | Low-cost stone or earth dams built by communities to store runoff, recharge aquifers, and support downstream use. | - Weir / wall represented as vector polyline - Crest level of the Weir above m.s.l (in meters) - An outline/polygon of expected ponded surface will be useful for validating mesh modification | - CN values and Manning’s n if changes are expected. - Weir / wall represented as vector polyline - Crest level of the Weir above m.s.l (in meters) - An outline/polygon of expected ponded surface will be useful for validating mesh modification |
| - P: Plains |  |  |  |  |
| P1 | Flood Embankments | Raised earthen or engineered barriers constructed along rivers and drains to prevent floodwater from spreading into settlements and farmlands. | - Weir / wall represented as vector polyline - Crest level of the Weir above m.s.l (in meters) if constant. Alternatively, height above terrain (meters above n.s.l) is needed | - Default: No hydrological implication - Elevation of the embankment (m) - Weir / wall represented as vector polyline - Optional: CN values and Manning’s n (only if changes are expected). |
| P2 | Embankment Reinforcement | Strengthening weak embankment sections using stone pitching, geo-bags, riprap, or concrete works to prevent breaches during high flows. | - Weir / wall represented as vector polyline - Crest level of the Weir above m.s.l (in meters) if constant. Alternatively, height above terrain (meters above n.s.l) is needed | - Weir / wall represented as vector polyline - Crest level of the Weir above m.s.l (in meters) if constant. Alternatively, height above terrain (meters above n.s.l) is needed - Optional: CN values and Manning’s n (only if changes are expected). |
| P3 | Relief Cuts | Controlled breaching or fuse-plug cuts designed to divert excess floodwater into designated areas, reducing pressure on critical embankments. | - Dimensions of the relief cut – final width, final bottom elevation in meters - Name of embankment and location of the relief cut (as vector point or vector polyline) | - Dimensions of the relief cut – final width, final bottom elevation in meters - Name of embankment and location of the relief cut (as vector point or vector polyline) |
| P4 | Pumping Stations at Drains | High-capacity pumps installed to remove trapped water from low-lying depressions and enhance drainage during prolonged flooding. | - Not modeled | - N/A |
| P5 | Rainwater Harvesting | Capturing and storing monsoon rainfall through ponds, tanks, or recharge wells to reduce runoff and enhance groundwater replenishment | Total volumetric storage of rainwater harvesting system. Note: cannot be modeled as an individual well/tank due to scale | - Default: No hydrological implication - Total volumetric storage of rainwater harvesting system. - Note: cannot be modeled as an individual well/tank due to scale |
| P6 | Retarding Basin | Designated temporary storage areas that hold excess floodwater and release it gradually, lowering downstream flood peaks. | - Weir / wall(s) represented as vector polyline - Crest level of the Weir/embankments above m.s.l (in meters) - An outline/polygon of expected ponded surface will be useful for validating mesh modification - Note: Any associated canal abstraction / diversions should be mentioned but may not be modeled due to scale | - CN values and Manning’s n if changes are expected. - Weir / wall(s) represented as vector polyline - Crest level of the Weir/embankments above m.s.l (in meters) - An outline/polygon of expected ponded surface will be useful for validating mesh modification - Note: Any associated canal abstraction / diversions should be mentioned but may not be modeled due to scale |
| P7 | Raising & Rehabilitation of Embankments | Increasing the height and structural integrity of existing embankments to improve flood protection under extreme rainfall scenarios. | - Weir / wall represented as vector polyline - Crest level of the Weir above m.s.l (in meters) if constant. Alternatively, height above terrain (meters above n.s.l) is needed | - Weir / wall represented as vector polyline - Crest level of the Weir above m.s.l (in meters) if constant. Alternatively, height above terrain (meters above n.s.l) is needed - Optional: CN values and Manning’s n (only if changes are expected). |
| P8 | Removing Anthropogenic Obstructions | Clearing man-made blockages encroachments, undersized culverts, siphons, and cross-drainage structures to restore natural drainage pathways. | - Note: Can be modeled only for linear floodplain obstructions, e.g. existing embankments, major roads/highways. - Vector (poly/line) of the hydraulic structures/ encorachments to be removed | - Note: Can be modeled only for linear floodplain obstructions, e.g. existing embankments, major roads/highways. - Vector (poly/line) of the hydraulic structures/ encorachments to be removed |
| P9 | Climate Smart-Agriculture Interventions | Adoption of resilient farming practices such as laser leveling, raised beds, zero tillage, drought-tolerant crops, and efficient irrigation systems. | - Not modeled | - Default: No hydrological implication |
| - H: Hamal Lake subsystem |  |  |  |  |
| H1 | Flood Protection Bunds | Perimeter embankments constructed around Hamal Lake to protect nearby villages and agricultural land from overflow and high flood levels. | - bund / wall represented as vector polyline - Crest level of the bund above m.s.l (in meters) if constant. Alternatively, height above terrain (meters above n.s.l) is needed | - Default: No hydrological implication - Optional: CN values and Manning’s n (only if changes are expected). - bund / wall represented as vector polyline - Crest level of the bund above m.s.l (in meters) if constant. Alternatively, height above terrain (meters above n.s.l) is needed |
| H2 | Guide Bunds | Engineered bunds placed strategically to direct and control the flow of floodwater entering or leaving the lake, reducing erosion and channel instability. | - bund / wall represented as vector polyline - Crest level of the bund above m.s.l (in meters) if constant. Alternatively, height above terrain (meters above n.s.l) is needed | - Optional: CN values and Manning’s n (only if changes are expected). - bund / wall represented as vector polyline - Crest level of the bund above m.s.l (in meters) if constant. Alternatively, height above terrain (meters above n.s.l) is needed |
| H3 | Escape Channels | Designated emergency drainage pathways that allow excess lake water to be safely conveyed toward downstream channels during extreme flood events. | - Please provide channel shape: cross- sections, slope, and flow capacity - Location: Vector polyline | - Manning’s n - Please provide channel shape: cross- sections, slope, and flow capacity - Please provide channel shape: cross- sections, slope, and flow capacity - Location: Vector polyline |
| H4 | Gabion Apron (Bed Stabilization) | Rock-filled wire mattresses installed along lake outlets and channels to prevent scouring, stabilize the bed, and protect structures from erosion. | - Not modeled | - N/A |
| H5 | Excavation of Flow Path / Drainage Area | Clearing and deepening of silted or blocked flow paths to improve drainage from Hamal Lake toward Aral and Manchar systems. | - Not modeled | - N/A |
| H6 | Large & Small Retention Ponds / Lakes | Creation of additional storage ponds to temporarily hold floodwater, reduce peak flows, and enhance controlled lake-level management. | - wall represented as vector polyline - volume of any major excavation leading to increase in water storage - Crest level of the Weir above m.s.l (in meters) - An outline/polygon of expected ponded surface will be useful for validating mesh modification Note: Any significant associated canal abstraction / diversions should be mentioned but may not be modeled due to scale | - Optional: CN values and Manning’s n if changes are expected. - wall represented as vector polyline - volume of any major excavation leading to increase in water storage - Crest level of the Weir above m.s.l (in meters) - An outline/polygon of expected ponded surface will be useful for validating mesh modification Note: Any significant associated canal abstraction / diversions should be mentioned but may not be modeled due to scale |
| ML: Manchar Lake and Indus |  |  |  |  |
| ML1 | Wetland Rehabilitation | Restoring degraded wetland areas around Manchar and the Indus floodplain to improve biodiversity, flood buffering, and water quality. | - Vector polygon outlining the location(s) of the intervention | - CN values and Manning’s n (only if changes are expected). |
| ML2 | Aamri Bridge Culvert | Upgrading and expanding culvert capacity at Aamri Bridge to improve water flow connectivity between Hamal–Aral–Manchar–Indus systems. | - Location as Vector point / line - A CAD drawing of the culvert outlining its dimensions | - Default: No hydrological implication - Location as Vector point / line - A CAD drawing of the culvert outlining its dimensions |
| ML3 | Remodeling of Aral Tail Channel | Deepening, widening, and clearing the Aral Tail Channel to increase drainage efficiency and reduce waterlogging around Manchar Lake | - A CAD drawing of the channel cross-sections outlining its dimensions - Peak design discharge capacity for model validation | - A CAD drawing of the channel cross-sections outlining its dimensions - Peak design discharge capacity for model validation |
| ML4 | Raising & Rehabilitation of Embankments | Strengthening and elevating protective embankments to prevent Indus backflow and reduce flood risk for surrounding communities. | - Vector (poly/line) of the embankment - If existing embankment, name of existing embankment - New elevation of the embankment | - Default: No hydrological implication |
| ML5 | Increased Capacity of Manchar Lake | Expanding the lake’s effective storage volume to hold excess floodwater, reduce downstream peak flows, and delay flood waves. | - Crest level of the bund above m.s.l (in meters) if constant. Alternatively, height above terrain (meters above n.s.l) is needed | - Default: No hydrological implication - Crest level of the bund, m.s.l (in meters) if constant. - Alternatively, height above terrain (meters above n.s.l) is needed |
| ML6 | Natural Wetland Interventions for Water Quality | Using reed beds, buffer strips, and natural filtration systems to reduce pollution loads and improve the ecological health of Manchar Lake. | - Vector polygon outlining the location(s) of the intervention | - CN values and Manning’s n (only if changes are expected). - Vector polygon outlining the location(s) of the intervention |
| ML7 | Solar-Powered Pumping Stations & Irrigation Systems | Deploying solar-driven pumps to improve drainage, support irrigation, and enhance climate-resilient water management around the lake. |  | - N/A |
| ML8 | Elevated Storage Tanks / Cross-Basin Water Transfer Systems | Installing elevated storage or transfer pipelines to move excess water to safer basins, reduce local flooding, and improve supply reliability |  | - N/A |
| ML9 | Restoration of Flood Plains (Encroachment Removal) | Clearing illegal encroachments and restoring natural floodplain corridors to enhance flood spread, storage, and river-channel resilience. | - Note: Can be modeled only for linear floodplain obstructions, e.g. existing embankments, major roads/highways. - Vector (poly/line) of the hydraulic structures/ encroachments to be removed | - Default: No hydrological implications - Note: Can be modeled only for linear floodplain obstructions, e.g. existing embankments, major roads/highways. - Vector (poly/line) of the hydraulic structures/ encroachments to be removed |