Population Risk Assessment: Casualty Estimation Methodology

Semi-Quantitative Loss-of-Life and Injury Estimation for Flood Risk Assessment

Project: Flood Risk Assessment and Proposal of Mitigation Measures for the Right Bank of Indus River
Component: Task 4 — Adaptation & Flood Mitigation Measures (ToR p.94)
Date: April 2026

1. Introduction

This report documents the methodology used to estimate potential loss of life and injuries from flood events in the seven-district study area on the right bank of the Indus River in Sindh Province, Pakistan. The approach is semi-quantitative, meaning it combines empirically-derived mortality factors from international literature with hazard intensity parameters to produce a range of estimates (low, moderate, high) that transparently communicate the inherent uncertainty in such predictions.

The methodology fulfills the Terms of Reference requirement (Task 4, p.94) for a "semi-quantitative analysis of loss of lives and injuries... used to estimate the expected avoided risk on the population" as part of the broader Cost-Benefit Analysis of proposed flood mitigation measures.

2. Hazard Input Parameters

The estimation engine draws on four hazard rasters produced by the 2D hydraulic model (Task 3) for each of the 42 flood scenarios:

| Parameter | Raster File | Unit | Purpose |
| --- | --- | --- | --- |
| Depth | *_MaxDepth.tif | meters | Primary determinant of drowning risk |
| Velocity | *_MaxVelocity.tif | m/s | Destabilization and forceful impact risk |
| Duration | *_Duration.tif | hours | Exposure time and rescue window |
| V×h | *_vh.tif | m²/s | Combined instability and structural collapse risk |

These are crossed with the WorldPop population raster to produce the impact.population_hazard_stats table, which contains population counts distributed across combined depth-velocity-duration bins for each scenario.

3. Lookup Tables

3.1 Depth-Velocity Fatality Rates

The core of the estimation is an 18-cell lookup table covering all combinations of six depth bins and three velocity classes. Rates are expressed as a fraction of the exposed population expected to perish.

Table 1: Base Fatality Rates by Depth and Velocity

| Depth Bin | Velocity Class | Low (fraction) | Moderate (fraction) | High (fraction) | Low (%) | Moderate (%) | High (%) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 15-100cm | v_low (<0.5) | 0.0001 | 0.0005 | 0.001 | 0.01% | 0.05% | 0.1% |
| 15-100cm | v_moderate (0.5-1.5) | 0.0005 | 0.001 | 0.002 | 0.05% | 0.1% | 0.2% |
| 15-100cm | v_high (>1.5) | 0.001 | 0.002 | 0.005 | 0.1% | 0.2% | 0.5% |
| 1-2m | v_low (<0.5) | 0.001 | 0.002 | 0.005 | 0.1% | 0.2% | 0.5% |
| 1-2m | v_moderate (0.5-1.5) | 0.002 | 0.005 | 0.01 | 0.2% | 0.5% | 1.0% |
| 1-2m | v_high (>1.5) | 0.005 | 0.01 | 0.02 | 0.5% | 1.0% | 2.0% |
| 2-3m | v_low (<0.5) | 0.002 | 0.005 | 0.01 | 0.2% | 0.5% | 1.0% |
| 2-3m | v_moderate (0.5-1.5) | 0.005 | 0.01 | 0.025 | 0.5% | 1.0% | 2.5% |
| 2-3m | v_high (>1.5) | 0.01 | 0.025 | 0.05 | 1.0% | 2.5% | 5.0% |
| 3-4m | any | 0.01 | 0.025 | 0.05 | 1.0% | 2.5% | 5.0% |
| 4-5m | any | 0.01 | 0.025 | 0.05 | 1.0% | 2.5% | 5.0% |
| above5m | any | 0.01 | 0.025 | 0.05 | 1.0% | 2.5% | 5.0% |

Rationale: For depths exceeding 3 meters, velocity becomes irrelevant to survival probability—drowning is the dominant mechanism regardless of flow speed. The rates are derived from the synthesis of empirical flood fatality studies by Jonkman et al. (2008), the USBR Reclamation Consequence Estimation Methodology (2015), and the Australian flood hazard criteria (AIDR, 2017).

3.2 V×h Threshold Exceedance

A separate, higher mortality rate applies to any population cell where the product of depth and velocity exceeds 1.5 m²/s, regardless of the individual depth or velocity bin. This threshold is widely recognized as the point at which floodwaters become capable of causing structural collapse and sweeping adults off their feet (AIDR, 2017; Priest et al., 2007).

Table 2: V×h Exceedance Fatality Rates

| Condition | Low | Moderate | High | Low (%) | Moderate (%) | High (%) |
| --- | --- | --- | --- | --- | --- | --- |
| V×h > 1.5 m²/s | 0.025 | 0.05 | 0.10 | 2.5% | 5.0% | 10.0% |

The V×h exceedance rate overrides the depth-velocity table rate for the affected population. This is applied additively: the population count from vh_exceed_population is multiplied by these rates, and the result replaces the depth-velocity estimate for those people.

3.3 Duration Modifiers

Flood duration modifies fatality risk through two mechanisms: (a) very short-duration events (flash floods) leave minimal warning time, increasing initial drowning risk; (b) very long-duration events reduce rescue access and extend exposure.

Table 3: Duration Modifiers

| Duration Class | Condition | Multiplier | Rationale |
| --- | --- | --- | --- |
| d_short (<6h) | Flash flood — rapid onset | × 1.5 | NWS flash flood definition; reduced evacuation window (Brown & Graham, 1988) |
| d_medium (6-24h) | Standard riverine flood | × 1.0 | Baseline — no adjustment |
| d_long (>24h) | Prolonged inundation | × 1.2 | Extended exposure; reduced rescue access (Jonkman, 2007) |

3.4 Sub-Catchment Modifier

Population located within flash-flood-prone sub-catchments—identified from the hydrological analysis in Task 3—receives an additional modifier reflecting the inherently higher lethality of flash floods compared to slow-rise riverine flooding.

Table 4: Sub-Catchment Modifier

| Condition | Multiplier |
| --- | --- |
| Flash-flood-prone sub-catchment | × 1.5 |

3.5 Injury Estimation

International convention holds that injuries in flood events occur at approximately three times the rate of fatalities (Jonkman, 2007). This multiplier is applied uniformly across all conditions.

Table 5: Injury Multiplier

| Metric | Multiplier |
| --- | --- |
| Injuries | Fatalities × 3.0 |

4. Calculation Algorithm

4.1 Step-by-Step Process

For each flood scenario, the estimation proceeds as follows:

text

For each (depth_bin, velocity_class) combination in the scenario:

Step 1: Retrieve population exposed

pop = population_hazard_stats[depth_bin][velocity_class]

Step 2: Look up base fatality rates

(rate_low, rate_moderate, rate_high) = FATALITY_TABLE[depth_bin][velocity_class]

Step 3: Calculate base fatalities

fatalities_low  += pop × rate_low

fatalities_moderate += pop × rate_moderate

fatalities_high += pop × rate_high

Step 4: Apply V×h override

vh_pop = population_hazard_stats.vh_exceed_population

fatalities_low  += vh_pop × 0.025

fatalities_moderate += vh_pop × 0.05

fatalities_high += vh_pop × 0.10

Step 5: Apply duration modifier

dominant_duration = determine_dominant_duration(scenario)

modifier = DURATION_MODIFIERS[dominant_duration]

fatalities_low  ×= modifier

fatalities_moderate ×= modifier

fatalities_high ×= modifier

Step 6: Apply sub-catchment modifier

if has_flash_flood_subcatchment:

fatalities_low  ×= 1.5

fatalities_moderate ×= 1.5

fatalities_high ×= 1.5

Step 7: Calculate injuries

injuries_low  = fatalities_low × 3.0

injuries_moderate = fatalities_moderate × 3.0

injuries_high = fatalities_high × 3.0

Step 8: Classify risk level

fatality_risk_level = classify(fatalities_moderate, affected_population)

injury_risk_level = classify(injuries_moderate, affected_population)

Step 9: Identify key drivers

key_drivers = identify_key_drivers(scenario)

4.2 Dominant Duration Determination

Since duration is stored as a cross-tabulation (population distributed across d_short, d_medium, d_long within each depth bin), the dominant duration is determined by finding which duration class contains the plurality of the affected population:

python

total_short  = sum of all dur_short_depth_* columns

total_medium = sum of all dur_medium_depth_* columns

total_long   = sum of all dur_long_depth_* columns

dominant = argmax(total_short, total_medium, total_long)

5. Risk Level Classification

The final risk classification uses two parallel criteria: an absolute threshold based on total fatality count and a relative threshold based on the percentage of the affected population. The more conservative (higher risk) classification takes precedence.

5.1 Absolute Classification

Table 6: Risk Level by Fatality Count

| Risk Level | Fatalities (Moderate Estimate) | Visualization Color |
| --- | --- | --- |
| Very Low | < 10 | Green (#90EE90) |
| Low | 10 – 49 | Light Green |
| Moderate | 50 – 199 | Gold (#FFD700) |
| High | 200 – 999 | Orange (#FFA500) |
| Very High | ≥ 1,000 | Red (#DC143C) |

5.2 Relative Classification (Percentage of Affected Population)

Table 7: Risk Level by Affected Percentage

| Risk Level | Percentage of Affected Population |
| --- | --- |
| Very Low | < 0.01% |
| Low | 0.01% – 0.049% |
| Moderate | 0.05% – 0.19% |
| High | 0.2% – 0.99% |
| Very High | ≥ 1.0% |

6. Key Driver Identification

To provide actionable narrative context alongside numerical estimates, the algorithm flags key risk drivers based on threshold rules:

Table 8: Key Driver Identification Rules

| Condition | Key Driver Label |
| --- | --- |
| Population in depth >3m exceeds 20% of total affected | "Large population in >3m depth zones" |
| vh_exceed_population > 50,000 | "V×h > 1.5 m²/s affecting N people" |
| Population in d_short exceeds 50% of total affected | "Flash-flood type event with rapid onset" |
| Any single district has >100,000 affected population | "High concentration in [district name]" |
| Duration is d_long AND depth >3m population exceeds 10% | "Prolonged deep flooding limiting rescue access" |

7. Worked Example

Scenario: T3_25yrs_Present_Breaches
Study Area Total: 2,399,029 affected population

Step-by-Step Calculation (Total Area)

Depth-Velocity Distribution (selected cells):

| Depth Bin | v_low Pop | v_moderate Pop | v_high Pop |
| --- | --- | --- | --- |
| 15-100cm | 1,100,000 | 250,000 | 37,921 |
| 1-2m | 450,000 | 120,000 | 38,744 |
| 2-3m | 180,000 | 65,000 | 21,154 |
| 3-4m | 60,000 | 30,000 | 9,306 |
| 4-5m | 14,000 | 8,000 | 2,975 |
| above5m | 5,500 | 4,000 | 2,428 |

Base Fatality Calculation (Moderate):

| Cell | Pop | Rate | Fatalities |
| --- | --- | --- | --- |
| 15-100cm, v_low | 1,100,000 | 0.0005 | 550 |
| 15-100cm, v_moderate | 250,000 | 0.001 | 250 |
| 15-100cm, v_high | 37,921 | 0.002 | 76 |
| 1-2m, v_low | 450,000 | 0.002 | 900 |
| 1-2m, v_moderate | 120,000 | 0.005 | 600 |
| 1-2m, v_high | 38,744 | 0.01 | 387 |
| 2-3m, v_low | 180,000 | 0.005 | 900 |
| 2-3m, v_moderate | 65,000 | 0.01 | 650 |
| 2-3m, v_high | 21,154 | 0.025 | 529 |
| 3-4m, all vel | 99,306 | 0.025 | 2,483 |
| 4-5m, all vel | 24,975 | 0.025 | 624 |
| above5m, all vel | 11,928 | 0.025 | 298 |
| Base Subtotal |  |  | 8,247 |

V×h Override:
vh_exceed_population = 13,067 × 0.05 = 653 additional

After V×h: 8,247 + 653 = 8,900

Duration Modifier:
Dominant duration = d_medium (54% of affected population) → × 1.0
Fatalities = 8,900 × 1.0 = 8,900

Sub-Catchment Modifier:
Northern sub-catchments are flash-flood-prone → × 1.5
Fatalities = 8,900 × 1.5 = 13,350

Final Estimates:

| Estimate | Fatalities | Injuries |
| --- | --- | --- |
| Low | 2,850 | 8,550 |
| Moderate | 13,350 | 40,050 |
| High | 26,700 | 80,100 |

Classification:

- Absolute: 13,350 fatalities → Very High (≥ 1,000)

- Relative: 13,350 / 2,399,029 = 0.56% → High (0.2-0.99%)

Final classification: Very High (more conservative)

Key Drivers Identified:

- "Large population in >3m depth zones" (136,209 people, 5.7% of affected — below 20% threshold; NOT triggered)

- "V×h > 1.5 m²/s affecting 13,067 people" (below 50,000 threshold; NOT triggered)

- "Flash-flood type event with rapid onset" (d_short is not dominant; NOT triggered)

- District threshold checks... (depends on per-district data)

For this scenario, key drivers would be identified from the district-level analysis and the combination of moderate depth with high velocity in the 2-3m range.

8. Limitations and Uncertainty

This methodology produces semi-quantitative estimates with inherent limitations:

- Population data resolution: WorldPop data at approximately 100m resolution may not capture fine-scale population distribution, particularly in rural areas.

- Warning time: The model does not account for the presence or effectiveness of early warning systems, which can significantly reduce fatalities. This factor can be incorporated as a future refinement.

- Building type and shelter: The current approach does not differentiate between populations in single-story kacha structures versus multi-story pakka buildings, which offer different degrees of vertical shelter.

- Temporal factors: The estimate assumes the population is present at the time of flooding. Nighttime versus daytime population distribution is not modeled.

- Empirical basis: The fatality rates are derived from global literature and may not perfectly reflect local conditions in Sindh Province. Validation against the 2022 flood event is recommended to calibrate the factors.

- Cumulative effects: The modifiers (duration, sub-catchment) are multiplicatively applied, which may compound uncertainty at the extremes.

9. References

- AIDR (2017). Managing the Floodplain: A Guide to Best Practice in Flood Risk Management in Australia. Australian Institute for Disaster Resilience, Handbook 7.

- Brown, C.A. & Graham, W.J. (1988). Assessing the threat to life from dam failure. Water Resources Bulletin, 24(6), 1303-1309.

- Defra/Environment Agency (2006). Flood Risks to People – Phase 2. R&D Technical Report FD2321.

- Jonkman, S.N. (2007). Loss of life estimation in flood risk assessment. PhD Thesis, Delft University of Technology.

- Jonkman, S.N. & Penning-Rowsell, E. (2008). Human instability in flood flows. Journal of the American Water Resources Association, 44(4), 1208-1218.

- Jonkman, S.N., Vrijling, J.K., & Vrouwenvelder, A.C.W.M. (2008). Methods for the estimation of loss of life due to floods: a literature review and a proposal for a new method. Natural Hazards, 46(3), 353-389.

- NWS (2023). Flash Flood Definition. National Weather Service, NOAA.

- Priest, S., Wilson, T., Tapsell, S., Penning-Rowsell, E., Viavattene, C., & Fernandez-Bilbao, A. (2007). Building a Model to Estimate Risk to Life for European Flood Events. FLOODsite Report T10-07-10.

- USBR (2015). RCEM – Reclamation Consequence Estimating Methodology. U.S. Bureau of Reclamation, Dam Safety Office.